Navi Robot — Hardware Interface Guide

| Section | Description |
| --- | --- |
| [Hardware Interface](getting-started/overview.md) | Complete hardware specifications for sensors, actuators, cameras, and compute modules. |
| [ROS Interface](ros-interface/environment.md) | Full ROS 1 (Noetic) topic and parameter reference for navigation, motion, and expressions. |
| [Audio System](audio/microphone.md) | Microphone capture, speaker output, PulseAudio and ROS-based volume control. |
| [System Services](services/overview.md) | Systemd services, boot sequence, and service management across both compute boards. |
| [Troubleshooting](troubleshooting.md) | Step-by-step solutions for connectivity, ROS, audio, camera, and service failures. |
| [Quick Reference](quick-reference.md) | Single-page cheat sheet with every command you need, organized by category. |

Platform Overview

The Navi Robot is a conversational rehabilitation and assistance robot built on a dual-board NVIDIA Jetson architecture, running ROS 1 (Noetic). This documentation provides hardware interface specifications for third-party developers building custom applications on the Navi platform.

[Confidential]  This document is intended solely for authorized integration partners. The proprietary Navi conversational AI application layer is not covered and is not accessible to integration partners.

Compute Architecture

| Component | Hardware | Primary Role |
| --- | --- | --- |
| Main Compute | NVIDIA Jetson AGX Orin | AI inference, conversational pipeline, vision models |
| ROS Controller | NVIDIA Jetson Orin NX | ROS master, navigation, motor control, sensor fusion |
| Microcontroller | STM32 | Low-level motor driver, cliff sensors, bumpers, RGB LEDs |
| Wireless MCU | ESP32 | Power management, head/neck servo control, digital I/O |

Document Scope

This guide covers:

Network topology and IP addressing for all onboard devices

ROS topic and parameter interfaces for sensors and actuators

Camera and microphone hardware access

Audio output and volume control

Navigation, head/neck, and expression control interfaces

System service architecture and boot sequence

Diagnostic and monitoring utilities

Document Info

| Field | Value |
| --- | --- |
| Platform | NVIDIA Jetson AGX Orin & Jetson Orin NX |
| ROS Version | ROS 1 (Noetic) |
| Document Version | 1.0 |
| Date | April 2026 |
| Organization | Navigait (Xavorcorp) |

Platform Overview

The Navi Robot is built on a dual-board NVIDIA Jetson architecture with a clear separation of concerns: one board handles ROS-based robot control, and the other handles AI inference and the conversational pipeline.

Hardware Architecture

┌─────────────────────────────────────────────────────┐
│                    NAVI ROBOT                       │
│                                                     │
│  ┌──────────────────┐    ┌──────────────────────┐   │
│  │  Jetson Orin NX  │    │  Jetson AGX Orin     │   │
│  │  (ROS Master)    │◄──►│  (AI Compute)        │   │
│  │  192.168.8.1     │    │  192.168.8.2         │   │
│  └────────┬─────────┘    └──────────────────────┘   │
│           │ Ethernet                                │
│  ┌────────▼──────┐                                  │
│  │ Ethernet      │                                  │
│  │ Switch        │                                  │
│  └───────────────┘                                  │
│           │                                         │
│  ┌────────▼───────┐                                 │
│  │  Robot Screen  │                                 │
│  │  (Display)     │                                 │
│  │  192.168.8.3   │                                 │
│  └────────────────┘                                 │
└─────────────────────────────────────────────────────┘

Compute Boards

Jetson Orin NX — ROS Controller

The Orin NX is the ROS Master and primary robot control board. It manages:

ROS Master node (192.168.8.1:11311)

Navigation stack and motor control

Sensor fusion (cliff sensors, bumpers, encoders)

Wi-Fi hotspot for development access

Cloud telemetry via cloud_comm.service

Jetson AGX Orin — AI Compute

The AGX Orin is a ROS Slave dedicated to AI workloads. It manages:

Conversational AI pipeline (chatbot.service)

NVIDIA Riva ASR/TTS (on-device speech)

Edge LLM inference (nano_llm_tools.service)

Video recording and vision models

IoT Hub bidirectional communication

Microcontrollers

The low-level microcontrollers are handling:

Wheel motor drivers

Fornt/ Rear Bumpers

RGB status LEDs

Communication with ROS via ROSSerial

Power management

Head/neck servo control

Internal IP Map

| Device | IP Address | Role |
| --- | --- | --- |
| Jetson Orin NX | `192.168.8.1` | ROS Master, Hotspot |
| Jetson AGX Orin | `192.168.8.2` | AI Compute (ROS Slave) |
| Robot Screen (Tablet) | `192.168.8.3` | Robot Facial Display |

[]  The 192.168.8.x subnet is reserved for the robot's internal network. Do not configure conflicting subnets on the home network.

Network & Connectivity

Internal Network Topology

All internal compute devices are connected via an Ethernet switch inside the robot chassis. The Jetson Orin NX serves as the ROS Master and provides a Wi-Fi hotspot for development access.

[ Cloud / Internet ]
                               │
                           Home Wi-Fi
                               │
                      ┌────────▼────────┐
                      │  Jetson Orin NX │◄─── Wi-Fi Hotspot (Dev Access)
                      │  192.168.8.1    │
                      └────────┬────────┘
                               │ Ethernet
                      ┌────────▼────────┐
                      │ Ethernet Switch │
                      └──┬──────────┬──┘
                         │          │
              ┌──────────▼──┐  ┌────▼──────────┐
              │ AGX Orin    │  │ Robot Screen  │
              │ 192.168.8.2 │  │ 192.168.8.3   │
              └─────────────┘  └───────────────┘

External Wi-Fi (Internet Access)

The robot connects to a home or commercial Wi-Fi network for cloud services.

| Feature | Detail |
| --- | --- |
| Supported bands | 2.4 GHz and 5 GHz |
| Authentication | WPA / WPA2 |
| Not supported | WEP, 802.1X enterprise |

[Offline Operation]  If external internet is unavailable, internal inter-device communication continues without interruption. All ROS traffic flows over the internal Ethernet switch.

Cloud Communication

The robot communicates with the cloud backend via Azure IoT Hub for telemetry, configuration sync, and remote commands.

| Service | Board | Purpose |
| --- | --- | --- |
| `cloud_comm.service` | Orin NX | Battery telemetry to IoT Hub |
| `iothub_monitor.service` | AGX Orin | Bidirectional command/status messaging |

Connection strings are pre-configured on each device — no manual setup is required for cloud connectivity.

Development Hotspot

The Orin NX broadcasts a Wi-Fi hotspot specifically for development and onboarding access.

[Production Note]  The hotspot connection is intended for development and onboarding only. In production, the robot connects to the user's home Wi-Fi for internet access. The internal Ethernet network remains active regardless of external Wi-Fi status.

SSH Access

Prerequisites

Connect your laptop to the robot's Wi-Fi hotspot before attempting SSH.

| Field | Value |
| --- | --- |
| SSID | `XVR_HOTSPOT_bd30d9` |
| Password | `Provided in the Email` |

[]  If the robot is connected to your Wi-Fi network, you can SSH into it via its WLAN IP address. To find the robot's WLAN IP, run ipconfig on the robot and look for the wireless interface address. In this case, use the WLAN IP for SSH while following the setups below.

Connecting to the Master Device (Orin NX)

The primary entry point for all development access is the Jetson Orin NX at 192.168.8.1.

ssh nvidia@192.168.8.1

| Field | Value |
| --- | --- |
| Host | `192.168.8.1` |
| Username | `Provided In Email` |
| Password | `Provided In Email` |

Connecting to the AI Compute Board (AGX Orin)

The AGX Orin is not directly exposed to the hotspot. SSH into it from the Orin NX:

# First, SSH into the Orin NX
ssh nvidia@192.168.8.1

# Then, from the Orin NX terminal, SSH into the AGX Orin
ssh nvidia@192.168.8.2

[]  You can also connect the AGX Orin directly to your Wi-Fi network using standard Linux CLI commands. Once connected, the device can be accessed over SSH via its WLAN IP, without needing to hop through the Orin NX. Ensure both your laptop and the AGX Orin are on the same Wi-Fi network.

Supported network types: WPA/WPA2 and Open networks. Enterprise security (e.g. 802.1X/PEAP) is not supported.

Steps to connect the AGX Orin to Wi-Fi:

1. SSH into the AGX Orin via the Orin NX (as shown above), then run the following commands:

2. List available Wi-Fi networks:

```bash

nmcli device wifi list

```

3. Connect to your Wi-Fi network:

```bash

# For WPA/WPA2 networks

sudo nmcli device wifi connect "YOUR_SSID" password "YOUR_PASSWORD"

# For open (password-free) networks

sudo nmcli device wifi connect "YOUR_SSID"

```

4. Verify the connection and find the assigned WLAN IP:

```bash

ip addr show wlan0

```

Once connected, use the displayed WLAN IP to SSH directly into the AGX Orin from any device on the same network:

```bash

ssh nvidia@<AGX_WLAN_IP>

```

VS Code Remote Development

You can also connect using VS Code Remote - SSH:

Install the Remote - SSH extension

Open the Command Palette → Remote-SSH: Connect to Host...

Enter nvidia@192.168.8.1

Enter password As provided in Email when prompted

ROS Environment

The .bashrc on each board automatically configures the ROS environment for every new terminal session:

| Variable | Value |
| --- | --- |
| `ROS_MASTER_URI` | `http://192.168.8.1:11311` |

[]  If you open a new terminal and ROS commands fail, verify the environment is sourced by running echo $ROS_MASTER_URI. It should return http://192.168.8.1:11311.

ROS Environment

Architecture Overview

Only the Jetson Orin NX runs ROS natively. It acts as the ROS Master for the entire robot.

The Jetson AGX Orin does not have ROS installed on the host. Instead, it communicates with the ROS ecosystem on OrinNX via the rosbridge server (WebSocket-based), using Python code that calls the roslibpy library. This allows the AGX Orin to publish/subscribe to topics and get/set parameters on the OrinNX without a native ROS installation.

| Board | ROS Access | Method |
| --- | --- | --- |
| Jetson Orin NX | Native ROS Noetic | Direct ROS environment |
| Jetson AGX Orin | Via rosbridge | `roslibpy` over WebSocket |

Orin NX — Native ROS Environment

ROS Master URI

http://192.168.8.1:11311

Environment Variables

The .bashrc on the Orin NX sources the ROS environment automatically on every terminal session:

| Variable | Value | Purpose |
| --- | --- | --- |
| `ROS_MASTER_URI` | `http://192.168.8.1:11311` | Identifies this device as the ROS Master |
| `ROS_HOSTNAME` | Local device hostname | Used for ROS networking |
| `ROS_IP` | Local device IP | Used for ROS networking |

Manual Sourcing

If you need to source the ROS environment manually on the Orin NX:

source /opt/ros/noetic/setup.bash

Verifying the Setup

Run these on the Orin NX (or any device with native ROS pointed at the master):

# Check ROS master is reachable
rostopic list

# Verify environment variables
echo $ROS_MASTER_URI
echo $ROS_HOSTNAME
echo $ROS_IP

AGX Orin — ROS Access via rosbridge

The AGX Orin does not run ROS on the host. To interact with ROS topics and parameters, it connects to the rosbridge WebSocket server running on the Orin NX.

rosbridge Endpoint

ws://192.168.8.1:9090

Python Example using roslibpy

import roslibpy

# Connect to the rosbridge server on OrinNX
client = roslibpy.Ros(host='192.168.8.1', port=9090)
client.run()

# Subscribe to a topic
listener = roslibpy.Topic(client, '/your/topic', 'std_msgs/String')
listener.subscribe(lambda msg: print(f"Received: {msg['data']}"))

# Publish to a topic
publisher = roslibpy.Topic(client, '/your/topic', 'std_msgs/String')
publisher.publish(roslibpy.Message({'data': 'hello from AGX Orin'}))

# Get a ROS parameter
param = roslibpy.Param(client, '/some_param')
print(param.get())

# Set a ROS parameter
param.set('new_value')

client.terminate()

[Install roslibpy on AGX Orin]  `bash pip install roslibpy `

[rosbridge must be running on OrinNX]  The rosbridge WebSocket server is started as part of the normal boot sequence on the Orin NX. If it is not reachable, verify rehab_boot.service is running on the Orin NX.

External Workstations (Native ROS)

If you have a workstation with ROS installed and want to connect it to the robot's ROS network via the hotspot:

export ROS_MASTER_URI=http://192.168.8.1:11311
export ROS_IP=<your-workstation-ip>
export ROS_HOSTNAME=<your-workstation-ip>

Then use standard ROS tools (rostopic, rosnode, etc.) as normal.

[]  Workstations without ROS can also use the rosbridge WebSocket endpoint (ws://192.168.8.1:9090) with roslibpy, just like the AGX Orin does.

Facial Expressions

Overview

The Navi robot displays facial expressions on its screen (the Robot Display Screen connected via Ethernet). Expressions are controlled by setting the /smiley_param ROS parameter, which is read by the facial display module and rendered on screen.

ROS Parameter

rosparam set /smiley_param <expression_name>

Available Expressions

| Expression Name | Description |
| --- | --- |
| `smiling` | Default friendly smile |
| `receiving_audio` | Listening / hearing indicator |
| `thinking` | Processing / thinking state |
| `talking` | Speaking indicator |
| `waving` | Greeting gesture |
| `lowbattery` | Low battery warning |
| `charging` | Charging in progress |

Examples

Smiling

rosparam set /smiley_param smiling

Listening

rosparam set /smiley_param receiving_audio

Thinking

rosparam set /smiley_param thinking

Talking

rosparam set /smiley_param talking

Low Battery

rosparam set /smiley_param lowbattery

Head & Neck Control

Overview

The robot's neck mechanism can be controlled either via joystick input or programmatically by setting the /neck_cmd ROS parameter. Control signals are processed by the ESP32 wireless MCU, which drives the head/neck servo.

ROS Parameter

rosparam set /neck_cmd <mode_integer>

Mode Reference

| Value | Behavior |
| --- | --- |
| `1` | Mean / Neutral Position |
| `2` | Mean / Neutral Position |
| `3` | Rotate Left |
| `4` | Rotate Right |

Examples

Neutral

rosparam set /neck_cmd 1

Rotate Left

rosparam set /neck_cmd 3

Rotate Right

rosparam set /neck_cmd 4

Verifying the Current Value

rosparam get /neck_cmd

Camera Systems

Overview

The robot is equipped with two camera systems, both connected via USB to the respective Jetson Device.

Camera Summary

| Camera | Connection | ROS Topic | Primary Use |
| --- | --- | --- | --- |
| Orbbec (Depth) | USB → AGX Orin | `/orbbec/camera/color/image_raw/compressed` | Activity recognition, depth sensing |
| Intel RealSense | USB → Orin NX | `/camera/color/image_raw/compressed` | Navigation, SLAM, obstacle avoidance |

Orbbec Depth Camera

The Orbbec camera provides both color and depth data. This camera is intended for AI models requiring the image access.

Accessing Orbbec Data

The Orbbec camera is connected to the AGX Orin device. To access camera data on the AGX Orin, use the rospylib in your python node.

rostopic echo /orbbec/camera/color/image_raw/compressed

Check Stream Frequency

rostopic hz /orbbec/camera/color/image_raw/compressed

Intel RealSense Camera

The RealSense camera is used for navigation.

Accessing RealSense Data

rostopic echo /camera/color/image_raw/compressed

Check Stream Frequency

rostopic hz /camera/color/image_raw/compressed

Important Notes

> AGX Orin — Accessing Camera Data:

> To access camera data on the AGX Orin device, you must use rospylib. Always subscribe to the compressed image topics (e.g., /orbbec/camera/color/image_raw/compressed) instead of the raw topics to achieve the highest possible frame rate.

Battery Monitoring

Overview

Battery status is available through two channels: a ROS topic for local access and Azure IoT Hub for remote cloud monitoring.

ROS Topic

| Field | Value |
| --- | --- |
| Topic | `/battery_display` |
| Message Type | `std_msgs/Float32` |
| Direction | Publish |
| Value Range | `0.0` – `100.0` (percentage) |

Check Battery Percentage

rostopic echo /battery_display

Monitor Continuously

# Watch battery level update in real time
rostopic echo /battery_display

Check Publish Frequency

rostopic hz /battery_display

Cloud Telemetry

Battery status is also reported to Azure IoT Hub via cloud_comm.service running on the Orin NX. This enables remote battery monitoring from the companion mobile app or backend dashboard, independent of a local ROS connection.

Low Battery Expression

When battery is low, the facial display expression can be set to lowbattery:

rosparam set /smiley_param lowbattery

The conversational AI pipeline handles this automatically when battery telemetry crosses a threshold.

Microphone Input

Overview

Audio is captured from the robot's hardware microphone via a GStreamer broadcaster pipeline. The audio-broadcaster.service runs at boot, captures the microphone using PulseAudio's echo-cancelled source, and fans the stream out to multiple UDP ports simultaneously using a tee element. This allows several independent services to consume the same audio without interfering with each other.

Prerequisites

Before using the microphone in your own code, ensure the following:

[Disable the default chatbot first]  The default chatbot.service and riva_container.service consume the audio ports. If they are running, your application will conflict with them. Stop them before connecting:

```bash

sudo systemctl stop chatbot.service

sudo systemctl stop riva_container.service

```

[Keep the broadcaster running]  Your application depends on audio-broadcaster.service being active. Do not stop it.

```bash

# Verify the broadcaster is running

systemctl status audio-broadcaster.service

# View live logs

journalctl -u audio-broadcaster.service -f

```

Port Allocation

The broadcaster sends identical copies of the audio stream to the following local UDP ports:

| Port | Service | Description |
| --- | --- | --- |
| `5000` | Raw Recorder | Raw audio for archival and custom processing |
| `5003` | VAD | Voice Activity Detection |

For custom integrations, connect to port `5000` unless you specifically need one of the other streams.

Audio Format

| Parameter | Value |
| --- | --- |
| Source | PulseAudio echo-cancelled source (`source_ec`) |
| Encoding | S16LE (16-bit signed little-endian PCM) |
| Channels | Mono (1 channel) |
| Sample Rate | 16,000 Hz |
| Transport | UDP (localhost) |

How the Broadcaster Works

The broadcaster uses a GStreamer tee element to duplicate the microphone signal and send it to all ports simultaneously:

import gi
gi.require_version('Gst', '1.0')
from gi.repository import Gst

Gst.init(None)

pipeline_str = (
    "pulsesrc device=source_ec ! "
    "audio/x-raw,channels=1,rate=16000,format=S16LE ! "
    "audioconvert ! audioresample ! "
    "tee name=t "
    "t. ! queue ! udpsink host=127.0.0.1 port=5000 "  # Raw Recording
    "t. ! queue ! udpsink host=127.0.0.1 port=5001 "  # Cloud ASR
    "t. ! queue ! udpsink host=127.0.0.1 port=5002 "  # Wake Word
)

This pipeline is managed by audio-broadcaster.service — you do not need to run it yourself.

Recording from the Stream

To capture audio from the broadcaster in your own application, connect a UDP socket to port 5000 and read the raw PCM bytes:

import socket
import wave

def record_audio(filename="output.wav", duration=5):
    UDP_IP = "127.0.0.1"
    UDP_PORT = 5000
    RATE = 16000  # Must match the broadcaster rate

    sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
    sock.bind((UDP_IP, UDP_PORT))

    frames = []
    print(f"Recording from UDP port {UDP_PORT} for {duration} seconds...")

    # Collect packets for the specified duration
    for _ in range(int(RATE / 1024 * duration * 2)):
        data, _ = sock.recvfrom(4096)
        frames.append(data)

    # Write collected frames to a WAV file
    with wave.open(filename, 'wb') as wf:
        wf.setnchannels(1)
        wf.setsampwidth(2)      # 16-bit = 2 bytes
        wf.setframerate(RATE)
        wf.writeframes(b''.join(frames))

    print(f"Saved to {filename}")

[Muting the microphone]  To mute the microphone at the OS level without stopping the broadcaster: `bash pactl set-source-mute @DEFAULT_SOURCE@ toggle `

Speaker Output

Overview

The Navi robot's speaker system is driven via Cloud TTS, streaming raw PCM audio directly to the hardware using the sounddevice library. This approach is chosen for two specific reasons:

Low latency — raw PCM skips MP3 decoding entirely

Instant interrupt — the stream can be aborted mid-sentence, unlike buffered players such as MPV

Prerequisites

Before using the speaker in your own code, ensure the following:

[Disable the default chatbot first]  The chatbot.service owns the speaker output at runtime. If it is running, your audio will conflict. Stop it before playing audio from your own code:

```bash

sudo systemctl stop chatbot.service

```

[Keep the broadcaster running]  The audio-broadcaster.service must remain active if your application also uses microphone input alongside speaker output.

```bash

systemctl status audio-broadcaster.service

```

Audio Format

| Parameter | Value |
| --- | --- |
| TTS Provider | ElevenLabs (`eleven_flash_v2_5` model) |
| Output Format | Raw PCM (`pcm_44100`) |
| Sample Rate | 44,100 Hz |
| Channels | Mono (1) |
| Playback Library | `sounddevice` |
| dtype | `float32` |

Hardware Path

AGX Orin (TTS Audio)
       │
   Audio Out (analog)
       │
  Audio Amplifier Board
       │
     Speakers

Playing TTS Audio

The following function streams text-to-speech from ElevenLabs and plays it directly to the speaker hardware. Audio is converted from 16-bit PCM bytes to float32 numpy arrays on the fly:

import numpy as np
import sounddevice as sd
from elevenlabs.client import ElevenLabs

def speak_and_play(text, api_key):
    client = ElevenLabs(api_key=api_key)

    # Request raw PCM — eliminates decoding lag
    audio_stream = client.text_to_speech.stream(
        text=text,
        voice_id="TbMNBJ27fH2U0VgpSNko",
        model_id="eleven_flash_v2_5",
        output_format="pcm_44100"
    )

    # Open the speaker hardware and stream chunks directly
    with sd.OutputStream(samplerate=44100, channels=1, dtype='float32') as stream:
        for chunk in audio_stream:
            # Convert 16-bit PCM bytes to float32 in [-1.0, 1.0]
            samples = np.frombuffer(chunk, dtype=np.int16).astype(np.float32) / 32768.0
            stream.write(samples.reshape(-1, 1))

Set your ElevenLabs API key via an environment variable rather than hardcoding it:

export ELEVENLABS_API_KEY="your_api_key_here"

import os
api_key = os.getenv("ELEVENLABS_API_KEY")
speak_and_play("Hello, I am Navi.", api_key)

Complete Record & Play Example

The following example ties microphone input and speaker output together: it records 5 seconds of audio from the broadcaster stream on port 5000, then plays back a confirmation response.

[Stop the chatbot before running this]  `bash sudo systemctl stop chatbot.service sudo systemctl stop riva_container.service `

import os
import socket
import wave
import numpy as np
import sounddevice as sd
from elevenlabs.client import ElevenLabs
from dotenv import load_dotenv

load_dotenv()
client = ElevenLabs(api_key=os.getenv("ELEVENLABS_API_KEY"))


def record_from_stream(output_file="test_record.wav", duration=5):
    """Listens to the broadcaster on port 5000 and saves audio to a WAV file."""
    UDP_IP = "127.0.0.1"
    UDP_PORT = 5000
    RATE = 16000  # Must match the broadcaster rate

    sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)
    sock.bind((UDP_IP, UDP_PORT))

    frames = []
    print(f"Recording from UDP port {UDP_PORT} for {duration} seconds...")

    for _ in range(int(RATE / 1024 * duration * 2)):
        data, _ = sock.recvfrom(4096)
        frames.append(data)

    with wave.open(output_file, 'wb') as wf:
        wf.setnchannels(1)
        wf.setsampwidth(2)  # 16-bit
        wf.setframerate(RATE)
        wf.writeframes(b''.join(frames))

    print(f"Saved to {output_file}")


def play_low_latency_tts(text):
    """Streams PCM from ElevenLabs directly to speakers via sounddevice."""
    print(f"Playing: {text}")

    audio_stream = client.text_to_speech.stream(
        text=text,
        voice_id="TbMNBJ27fH2U0VgpSNko",
        model_id="eleven_flash_v2_5",
        output_format="pcm_44100"
    )

    with sd.OutputStream(samplerate=44100, channels=1, dtype='float32') as stream:
        for chunk in audio_stream:
            samples = np.frombuffer(chunk, dtype=np.int16).astype(np.float32) / 32768.0
            stream.write(samples.reshape(-1, 1))


if __name__ == "__main__":
    record_from_stream("user_input.wav", duration=5)
    play_low_latency_tts("I have finished recording your audio from the system stream.")

Volume Control — Linux (PulseAudio)

Overview

Volume on the Navi robot is managed at the OS level via PulseAudio. This is the recommended method for application-level volume control. All commands use the DISPLAY= prefix because the robot runs headless (no graphical session).

[Use This for Application-Level Control]  For any application that needs to adjust volume, use these PulseAudio commands. Do not use the ROS audio topics for application-level volume — those control the hardware amplifier directly and can damage speaker hardware.

Set Volume (Absolute)

Set the volume of sink 0 (primary audio output) to a specific percentage:

DISPLAY= pactl set-sink-volume 0 <percentage>%

Examples:

# Set volume to 10%
DISPLAY= pactl set-sink-volume 0 10%

# Set volume to 50%
DISPLAY= pactl set-sink-volume 0 50%

# Set volume to 100%
DISPLAY= pactl set-sink-volume 0 100%

Adjust Volume (Relative)

Increase or decrease volume relative to the current level:

# Increase by 5%
DISPLAY= pactl set-sink-volume 0 +5%

# Decrease by 5%
DISPLAY= pactl set-sink-volume 0 -5%

Retrieve Current Volume

Get the current volume level as a clean percentage string:

DISPLAY= pactl list sinks | grep 'Volume:' | head -n 1 | awk -F / '{print $2}' | xargs

How it works:

| Command | Purpose |
| --- | --- |
| `pactl list sinks` | Dumps all audio output device info |
| `grep 'Volume:'` | Filters to volume lines only |
| `head -n 1` | Takes the first occurrence (default device) |
| `awk -F / '{print $2}'` | Extracts the percentage value |
| `xargs` | Trims surrounding whitespace |

Mute Audio

DISPLAY= pactl set-sink-mute 0 1

| Value | Effect |
| --- | --- |
| `1` (or `true`) | Mute on |
| `0` (or `false`) | Mute off |

Unmute Audio

DISPLAY= pactl set-sink-mute 0 0

[]  Unmuting restores volume to whatever level it was at before muting.

Toggle Mute State

DISPLAY= pactl set-sink-mute 0 toggle

This switches between muted and unmuted — the most convenient command for a mute/unmute button implementation.

Mute Default Microphone

pactl set-source-mute @DEFAULT_SOURCE@ toggle

The @DEFAULT_SOURCE@ shortcut avoids needing to look up the full microphone device name.

Command Reference Summary

| Action | Command |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Set volume to 50% | `DISPLAY= pactl set-sink-volume 0 50%` |  |  |  |  |
| Increase by 5% | `DISPLAY= pactl set-sink-volume 0 +5%` |  |  |  |  |
| Decrease by 5% | `DISPLAY= pactl set-sink-volume 0 -5%` |  |  |  |  |
| Get current volume | `DISPLAY= pactl list sinks \ | grep 'Volume:' \ | head -n 1 \ | awk -F / '{print $2}' \ | xargs` |
| Mute | `DISPLAY= pactl set-sink-mute 0 1` |  |  |  |  |
| Unmute | `DISPLAY= pactl set-sink-mute 0 0` |  |  |  |  |
| Toggle mute | `DISPLAY= pactl set-sink-mute 0 toggle` |  |  |  |  |
| Toggle mic mute | `pactl set-source-mute @DEFAULT_SOURCE@ toggle` |  |  |  |  |

Volume Control — ROS Interface

Overview

ROS-based volume control topics provide programmatic control of the robot's audio hardware from within the ROS ecosystem. These topics interface directly with the hardware amplifier.

[Hardware Amplifier Control — Internal Use Only]  The ROS audio topics control the hardware amplifier level directly. This interface is reserved for NaviGait's internal use only and should never be used at the application level. Incorrect amplifier settings can damage speaker hardware.

For application-level volume control, use the [PulseAudio commands](volume-linux.md) instead.

Topics

Topic

`/audio_mute_control`

`/audio_volume_control`

`/audio_power_control`

When to Use Each Interface

| Scenario | Use |
| --- | --- |
| Application adjusting playback volume | [PulseAudio (Linux)](volume-linux.md) |
| NaviGait internal amplifier calibration | ROS topics (this page) |
| Muting microphone input | `pactl set-source-mute @DEFAULT_SOURCE@ toggle` |

Services Overview

Architecture

The Navi robot runs two separate compute boards, each with its own set of systemd services. Services are configured to start automatically on boot in a defined sequence.

| Board | IP | Primary Role |
| --- | --- | --- |
| Jetson Orin NX | `192.168.8.1` | ROS control, navigation, sensors |
| Jetson AGX Orin | `192.168.8.2` | AI inference, conversational pipeline |

All Services at a Glance

Orin NX (ROS Controller Board)

| Service | Description | Restart Policy |
| --- | --- | --- |
| `hotspot.service` | Initializes the Jetson wireless hotspot for remote access | On boot |
| `rehab_boot.service` | Launches robotic and autonomous functionalities (navigation, motor control, sensor drivers) | On boot |
| `cloud_comm.service` | Manages IoT Hub telemetry including battery status payloads | On boot |

AGX Orin (AI Compute Board)

| Service | Description | Restart Policy |
| --- | --- | --- |
| `chatbot.service` | Runs the conversational AI agent (central intelligence module) | Always |
| `audio-broadcaster.service` | GStreamer audio pipelines for microphone capture and broadcast | Always |
| `riva_container.service` | NVIDIA Riva container for on-device ASR and TTS | Always |
| `nano_llm_tools.service` | Edge-optimized LLM inference container | Always |
| `iothub_monitor.service` | Bidirectional IoT Hub communication for commands and telemetry | Always |
| `video-recorder.service` | Camera hardware initialization and video recording management | Always |
| `robot-sync-api.service` | Fetches latest cloud configuration at boot (oneshot) | No (oneshot) |

Service Management Commands

# Check service status
systemctl status <service-name>

# View live service logs
journalctl -u <service-name> -f

# Restart a service
sudo systemctl restart <service-name>

# Enable a service (persist across reboots)
sudo systemctl enable <service-name>

# Disable a service (testing only — re-enable before reboot)
sudo systemctl disable <service-name>

[Critical Warning]  Do not modify or disable systemd service configurations. If any service is stopped for testing, ensure it is re-enabled before the next system reboot. Failure to do so may result in incomplete system initialization and loss of core functionality.

[Running a Custom Conversation Pipeline]  If you want to run your own pipeline of conversation, make sure that on the AGX Orin all services are disabled except audio-broadcaster.service, which is responsible for running the microphone. All other AGX Orin services must be stopped to avoid conflicts with your custom pipeline.

Orin NX Services

The Jetson Orin NX is the ROS Master board. It runs all core robot control services.

hotspot.service

| Field | Detail |
| --- | --- |
| Purpose | Initializes the Jetson wireless hotspot for remote SSH and ROS access |
| Restart Policy | On boot |
| Board | Jetson Orin NX |

systemctl status hotspot.service
journalctl -u hotspot.service -f

rehab_boot.service

| Field | Detail |
| --- | --- |
| Purpose | Launches primary robotic and autonomous functionalities |
| Restart Policy | On boot |
| Board | Jetson Orin NX |

systemctl status rehab_boot.service
journalctl -u rehab_boot.service -f

cloud_comm.service

| Field | Detail |
| --- | --- |
| Purpose | Manages IoT Hub telemetry including battery status payloads |
| Restart Policy | On boot |
| Board | Jetson Orin NX |

systemctl status cloud_comm.service
journalctl -u cloud_comm.service -f

AGX Orin Services

The Jetson AGX Orin is the AI Compute board. It runs all services related to the conversational AI pipeline, speech, and cloud communication.

chatbot.service

| Field | Detail |
| --- | --- |
| Purpose | Runs the Navi conversational AI agent — the central intelligence module |
| Dependencies | `nano_llm_tools.service`, `riva_container.service`, `audio-broadcaster.service`, `video-recorder.service` |
| Restart Policy | Always (auto-restart on failure) |
| Start Order | Phase 3 — after all upstream dependencies are healthy |

systemctl status chatbot.service
journalctl -u chatbot.service -f

audio-broadcaster.service

| Field | Detail |
| --- | --- |
| Purpose | Activates GStreamer pipelines to capture and broadcast microphone streams |
| Dependencies | System audio hardware (ALSA/PulseAudio) |
| Restart Policy | Always |
| Start Order | Phase 1 (early boot) — required before `chatbot.service` |

Makes raw audio input available to ASR, wake word detection, and voice activity detection modules over GStreamer's UDP streaming infrastructure.

systemctl status audio-broadcaster.service
journalctl -u audio-broadcaster.service -f

riva_container.service

| Field | Detail |
| --- | --- |
| Purpose | Launches the NVIDIA Riva container for on-device ASR and TTS |
| Dependencies | NVIDIA Container Runtime, GPU drivers |
| Restart Policy | Always |
| Start Order | Phase 1 (early boot) — must be healthy before `chatbot.service` starts |

systemctl status riva_container.service
journalctl -u riva_container.service -f

nano_llm_tools.service

| Field | Detail |
| --- | --- |
| Purpose | Starts the Nano LLM container for edge-optimized LLM inference |
| Dependencies | NVIDIA Container Runtime, GPU drivers |
| Restart Policy | Always |
| Start Order | Phase 1 (early boot) — must be healthy before `chatbot.service` starts |

systemctl status nano_llm_tools.service
journalctl -u nano_llm_tools.service -f

iothub_monitor.service

| Field | Detail |
| --- | --- |
| Purpose | Bidirectional IoT Hub communication between the robot and cloud backend |
| Dependencies | Network connectivity, Azure IoT Hub credentials |
| Restart Policy | Always |
| Start Order | Phase 2 — after network is available |

systemctl status iothub_monitor.service
journalctl -u iothub_monitor.service -f

video-recorder.service

| Field | Detail |
| --- | --- |
| Purpose | Activates camera hardware and manages video recording sessions |
| Dependencies | Camera hardware (CSI/USB), ffmpeg |
| Restart Policy | Always |
| Start Order | Phase 1 — independent of conversational pipeline |

Initializes the camera feed for activity recognition, gait analysis, and fall detection. Handles on-demand video capture, encoding via ffmpeg, and local storage.

systemctl status video-recorder.service
journalctl -u video-recorder.service -f

Orbbec Camera Container

The Orbbec depth camera driver runs via a Docker container (aliahmadxvr/orbbec-noetic:jetpack5.1.2) directly on boot using --restart=always. You can disable it if the camera is not required.

Manual Run (Interactive/Debugging)

Use this command to run the container in interactive mode. The --rm flag ensures the container is automatically removed when stopped.

sudo docker run -it --rm \
  --name orbbec-container \
  --network=host \
  -e ROS_MASTER_URI=http://192.168.8.1:11311 \
  -e ROS_HOSTNAME=$(hostname -I | awk '{print $1}') \
  -e ROS_IP=$(hostname -I | awk '{print $1}') \
  --runtime nvidia \
  --device /dev/bus/usb:/dev/bus/usb \
  --privileged \
  aliahmadxvr/orbbec-noetic:jetpack5.1.2

Run on Boot (Automatic Startup)

To ensure the camera driver and ROS topics are published automatically on every boot, use the command below. The --restart=always flag lets Docker manage automatic startup.

sudo docker run -d \
  --name orbbec-container \
  --network=host \
  -e ROS_MASTER_URI=http://192.168.8.1:11311 \
  -e ROS_HOSTNAME=$(hostname -I | awk '{print $1}') \
  -e ROS_IP=$(hostname -I | awk '{print $1}') \
  --runtime nvidia \
  --device /dev/bus/usb:/dev/bus/usb \
  --privileged \
  --restart=always \
  aliahmadxvr/orbbec-noetic:jetpack5.1.2

Operational Guidelines

Critical Rules

[Do Not Modify Systemd Services]  Never modify the existing systemd service configurations. Changes to service unit files can break the boot sequence and leave the robot in an unrecoverable state without a manual reboot and repair.

[Always Re-enable Services Before Reboot]  If you disable a service for testing, you must re-enable it before rebooting. Leaving core services disabled will result in incomplete system initialization. A robot that boots without its core services may not be reachable or functional.

[Do Not Conflict with the Internal Subnet]  The robot's internal IP space (192.168.8.x) is reserved. Do not configure conflicting subnets on the home or lab network.

Development Best Practices

| Practice | Guidance |
| --- | --- |
| Debugging | Use `rostopic list` and `rostopic echo` for real-time data inspection |
| Battery | Monitor `/battery_display` regularly during extended testing sessions |
| SSH sessions | Use tmux or screen to keep sessions alive during long operations |
| Service logs | Always check `journalctl -u <service> -f` when a service misbehaves |
| Network | Verify `ROS_MASTER_URI`, `ROS_IP`, and `ROS_HOSTNAME` are set correctly in each terminal |

Recommended Debugging Workflow

Check if ROS is running:

```bash

rostopic list

```

Check service health:

```bash

systemctl status rehab_boot.service

systemctl status chatbot.service

```

Check recent logs:

```bash

journalctl -u rehab_boot.service --since "5 minutes ago"

```

Verify network:

```bash

ping 192.168.8.1

ping 192.168.8.2

```

Check battery:

```bash

rostopic echo /battery_display

```

Troubleshooting

Common problems and their solutions when working with the Navi robot platform.

Connectivity Issues

Cannot SSH into the robot

Symptoms: ssh: connect to host 192.168.8.1 port 22: Connection refused or Network is unreachable

Checklist:

Confirm your workstation is connected to the robot's Wi-Fi hotspot — not your home network.

Ping the Orin NX first:

```bash

ping 192.168.8.1

```

If ping fails, the hotspot may not have started. The hotspot.service may have failed at boot. Physically access the robot and check:

```bash

systemctl status hotspot.service

journalctl -u hotspot.service -n 30

```

Try restarting the hotspot service:

```bash

sudo systemctl restart hotspot.service

```

ROS Issues

rostopic list returns nothing or hangs

Cause: The ROS Master is not reachable, or the ROS environment is not sourced correctly.

Fix:

# Verify environment
echo $ROS_MASTER_URI
# Expected: http://192.168.8.1:11311

# If empty, source manually
source /opt/ros/noetic/setup.bash

# Test master connectivity
rostopic list

If the master is unreachable, check rehab_boot.service:

systemctl status rehab_boot.service
journalctl -u rehab_boot.service -n 50

Audio Issues

No sound from the robot

Checklist:

Check the current volume is not zero:

```bash

DISPLAY= pactl list sinks | grep 'Volume:' | head -n 1 | awk -F / '{print $2}' | xargs

```

If volume reads 0%, set it to an audible level:

```bash

DISPLAY= pactl set-sink-volume 0 60%

```

Check if audio is muted:

```bash

DISPLAY= pactl list sinks | grep 'Mute:'

# If "Mute: yes", unmute:

DISPLAY= pactl set-sink-mute 0 0

```

Check audio-broadcaster.service is running:

```bash

systemctl status audio-broadcaster.service

```

Microphone input is not being captured

Checklist:

Confirm audio-broadcaster.service is active:

```bash

systemctl status audio-broadcaster.service

journalctl -u audio-broadcaster.service -n 30

```

Check if the microphone is muted:

```bash

pactl list sources | grep -A 5 'Name:'

```

Unmute the default microphone:

```bash

pactl set-source-mute @DEFAULT_SOURCE@ 0

```

Camera Issues

Camera topics show no data

Orbbec camera:

systemctl status orbbec_camera.service
journalctl -u orbbec_camera.service -n 30

Check logs at:

~/camera_service_output.log

~/camera_service_error.log

RealSense camera:

The RealSense is managed by the rehab_boot.service launch file. If this service is healthy but /camera/color/image_raw shows no data, check if the USB connection is secure.

# Check if camera device is detected
lsusb | grep -i intel
lsusb | grep -i orbbec

Battery Issues

Battery percentage not publishing

# Check if topic exists
rostopic list | grep battery

# Check cloud_comm service
systemctl status cloud_comm.service
journalctl -u cloud_comm.service -n 30

Logs at:

~/cloud_comm_output.log

~/cloud_comm_error.log

Cloud Connectivity Issues

IoT Hub not receiving telemetry

Checklist:

Confirm the robot is connected to home Wi-Fi (internet access):

```bash

ping -c 3 8.8.8.8

```

Check iothub_monitor.service on the AGX Orin:

```bash

systemctl status iothub_monitor.service

journalctl -u iothub_monitor.service -n 30

```

Check cloud_comm.service on the Orin NX:

```bash

systemctl status cloud_comm.service

```

General Recovery

If the robot is in an unknown state and you cannot diagnose the issue remotely, perform a clean reboot:

# On Orin NX
sudo reboot

# On AGX Orin (SSH from Orin NX)
ssh nvidia@192.168.8.2
sudo reboot

All systemd services will restart automatically in the correct order. Allow 2–3 minutes for full initialization before testing again.

[Before Rebooting]  Ensure all services you may have disabled during testing are re-enabled before rebooting: `bash sudo systemctl enable chatbot.service sudo systemctl enable riva_container.service sudo systemctl enable nano_llm_tools.service `

Quick Reference

A condensed reference card for common operations on the Navi robot platform.

SSH Access

| Target | Command |
| --- | --- |
| ROS Master (Orin NX) | `ssh nvidia@192.168.8.1` |
| AI Board (AGX Orin) | `ssh nvidia@192.168.8.2` (from Orin NX) |
| Default password | `Provided in Email` |

Head & Neck

| Action | Command |
| --- | --- |
| Neutral position | `rosparam set /neck_cmd 1` |
| Rotate left | `rosparam set /neck_cmd 3` |
| Rotate right | `rosparam set /neck_cmd 4` |

Facial Expressions

| Action | Command |
| --- | --- |
| Set expression | `rosparam set /smiley_param <expression>` |
| Available expressions | `smiling`, `receiving_audio`, `thinking`, `talking`, `waving`, `lowbattery`, `charging` |

Sensors & Diagnostics

| Action | Command |
| --- | --- |
| List all ROS topics | `rostopic list` |
| Check battery | `rostopic echo /battery_display` |
| RealSense camera stream | `rostopic echo /camera/color/image_raw` |
| Echo any topic | `rostopic echo /<topic_name>` |
| Check topic frequency | `rostopic hz /<topic_name>` |

Audio — Volume Control (PulseAudio)

| Action | Command |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Set volume to 50% | `DISPLAY= pactl set-sink-volume 0 50%` |  |  |  |  |
| Increase volume by 5% | `DISPLAY= pactl set-sink-volume 0 +5%` |  |  |  |  |
| Decrease volume by 5% | `DISPLAY= pactl set-sink-volume 0 -5%` |  |  |  |  |
| Get current volume | `DISPLAY= pactl list sinks \ | grep 'Volume:' \ | head -n 1 \ | awk -F / '{print $2}' \ | xargs` |
| Mute | `DISPLAY= pactl set-sink-mute 0 1` |  |  |  |  |
| Unmute | `DISPLAY= pactl set-sink-mute 0 0` |  |  |  |  |
| Toggle mute | `DISPLAY= pactl set-sink-mute 0 toggle` |  |  |  |  |
| Toggle mic mute | `pactl set-source-mute @DEFAULT_SOURCE@ toggle` |  |  |  |  |

Service Management

| Action | Command |
| --- | --- |
| Check service status | `systemctl status <service-name>` |
| View service logs | `journalctl -u <service-name> -f` |
| Restart a service | `sudo systemctl restart <service-name>` |
| Enable a service | `sudo systemctl enable <service-name>` |

IP Address Map

| Device | IP |
| --- | --- |
| Jetson Orin NX (ROS Master) | `192.168.8.1` |
| Jetson AGX Orin (AI) | `192.168.8.2` |
| Android Tablet (Robot Screen) | `192.168.8.3` |